Capstone Project Proposal

# Design and Evaluation of a Deception-in-Depth Honeynet Architecture for Observing Multi-Stage Attacker Behaviour

Contents

Executive Summary	4

1. Introduction	4

2. Project Objectives	5

3. Project Scope	5

3.1 Research Questions	6

3.2 Study Hypothesis	6

4. System Architecture	7

4.1 Telemetry Instrumentation	8

4.2 Interaction Sequence Reconstruction	8

4.3 Attacker Path Visualization	8

4.4 Deception Control Plane	9

5. Experimental Design	9

6. Behavioural Metrics	10

6.1 Evaluation Method	10

6.2 Threat Model	11

7. MITRE ATT&CK Mapping	11

8. Feasibility and Risk Management	11

9. Project Schedule and Learning Hours	12

10. Expected Deliverables	12

11. Expected Contributions	13

Appendix A – Implementation Plan	14

A.1 Purpose	14

A.2 Deployment Platform	14

A.3 Planned Virtual Machine Layout	14

A.4 Rationale for Feasibility	15

A.5 Proposed Network Layout	16

A.6 Logical Traffic Flow	16

A.7 Planned Role of Each VM	17

A.8 Deception Asset Placement Plan	19

A.9 Implementation Sequence	20

A.10 Snapshot and Reset Strategy	21

A.11 Key Manageability Considerations	21

A.12 Summary	22

Appendix B – Experimental Telemetry Schema	22

B.1 Purpose	22

B.2 Logging Architecture Overview	23

B.3 Core Telemetry Datasets	23

B.4 Session Log	24

B.5 Event Log	24

B.6 Artifact Interaction Log	25

B.7 Interaction Edge Log	26

B.8 Control Plane Trigger Log	27

B.9 Attacker Path Reconstruction	28

B.10 Alignment with Behavioural Metrics	28

Appendix C – Telemetry Processing Pipeline	29

C.1 Purpose	29

C.2 Pipeline Architecture	29

C.3 Log Sources	30

C.4 Event Normalization	30

C.5 Session Identification	31

C.6 Artifact Identification	31

C.7 Interaction Edge Generation	32

C.8 Structured Data Export	32

C.9 Analysis and Visualization	33

C.10 Implementation Considerations	33

# Executive Summary

Cyber deception technologies such as honeypots are widely used to observe malicious activity and study attacker behaviour within controlled environments. However, many honeypot deployments simulate only isolated services and therefore provide limited visibility into attacker activity after initial access.

This project proposes the design and evaluation of a Deception-in-Depth honeynet architecture that integrates deception mechanisms across multiple infrastructure layers, including network-level honeypots, host-level enterprise deception, and data-level decoy assets.

The system will simulate a small enterprise environment using virtualization technology. Attackers interacting with the environment will encounter multiple deceptive assets intended to influence their navigation and behaviour. To support structured behavioural observation, the architecture introduces a Deception Control Plane, which conditionally activates deeper deception layers when specific attacker actions are detected.

The project evaluates attacker interaction using three behavioural indicators:

- misdirection – interaction with deception artifacts

- delay – time spent navigating deceptive paths

- engagement depth – progression through multiple layers of the environment

Telemetry collected from the honeynet will be normalized into structured interaction records that allow reconstruction of attacker navigation paths and identification of interactions with deception artifacts.

Observed activities will also be mapped to MITRE ATT&CK tactics to provide structured interpretation of attacker behaviour.

The environment will be deployed on a Proxmox-based virtualization platform using approximately four to six virtual machines representing honeypot services, enterprise hosts, and monitoring infrastructure.

The final deliverables include a working layered honeynet environment, a documented experimental methodology, and behavioural analysis of observed attacker activity.

The project prioritizes experimental clarity, reproducibility, and manageable system scale, ensuring feasibility within the available hardware resources and capstone timeframe.

# 1. Introduction

Cyber deception technologies such as honeypots are widely used to observe malicious activity and collect intelligence about attacker behaviour. Traditional honeypots typically simulate a single vulnerable service and record interactions with that service. While effective for detecting automated attacks, isolated honeypots provide limited insight into attacker behaviour once access to the system has been established.

In real intrusion scenarios, attackers often perform multiple stages of activity including reconnaissance, credential discovery, lateral movement, and data access. Recent research suggests that layered deception architectures, where deception mechanisms exist across different infrastructure layers, can provide richer behavioural insights into attacker decision-making.

This project proposes the design and evaluation of a Deception-in-Depth honeynet architecture that integrates deception across network services, enterprise hosts, and data-level assets.

The honeynet environment will simulate a small enterprise network and allow researchers to observe attacker behaviour across multiple stages of activity. Behavioural indicators such as misdirection, delay, and engagement depth will be used to evaluate how deception influences attacker navigation decisions.

The experimental design is guided by a set of research questions focusing on how layered deception influences attacker navigation, engagement, and decision-making within the honeynet environment.

The project emphasizes controlled experimentation and behavioural observation rather than large-scale production deployment.

# 2. Project Objectives

The primary objective of this project is to design, implement, and evaluate a layered honeynet environment capable of observing multi-stage attacker behaviour.

Objective 1 – Design a layered deception architecture

Develop an experimental honeynet architecture integrating deception mechanisms across network, host, and data layers.

Objective 2 – Implement a controlled honeynet environment

Deploy the architecture within a virtualized infrastructure using open-source tools and simulated enterprise hosts.

Objective 3 – Capture multi-stage attacker interactions

Collect telemetry describing attacker behaviour including scanning, login attempts, command execution, and access to deception artifacts.

Objective 4 – Evaluate behavioural impact of layered deception

Measure how deception influences attacker navigation using behavioural metrics including misdirection, delay, and engagement depth.

Objective 5 – Interpret attacker behaviour using MITRE ATT&CK

Map observed activities to high-level MITRE ATT&CK tactics to provide structured interpretation of the behaviour captured during experiments.

# 3. Project Scope

The project focuses on constructing a controlled honeynet environment for behavioural observation rather than developing a production security platform.

The scope includes:

- designing a layered deception architecture

- deploying the environment using virtualization

- collecting telemetry from attacker interaction

- analysing behavioural patterns across deception layers

The project does not attempt to simulate a full enterprise infrastructure or implement advanced detection systems. Instead, the emphasis is on controlled experimentation and behavioural observation.

## 3.1 Research Questions

The study is guided by the following research questions.

RQ1 – How does layered deception influence attacker navigation within the honeynet environment?

This question examines whether attackers interact with additional systems beyond the initial honeypot when deeper deception layers are present.

RQ2 – To what extent do deception artifacts influence attacker decisions?

This question investigates whether artifacts such as decoy credentials or documents lead attackers to attempt credential reuse or access additional hosts.

RQ3 – Does staged activation of deception layers increase attacker engagement?

This question evaluates whether progressive activation of deception layers results in deeper exploration of the environment.

RQ4 – How does layered deception affect the time attackers spend interacting with the system?

This question examines whether deception mechanisms introduce measurable delays in attacker progression.

RQ5 – What attacker activities can be observed across different stages of the attack lifecycle?

This question focuses on identifying behavioural patterns and mapping observed actions to MITRE ATT&CK tactics.

## 3.2 Study Hypothesis

The central hypothesis of this study is that a staged Deception-in-Depth architecture will encourage attackers to interact more extensively with deceptive assets and explore more layers of the environment than a single-layer honeypot deployment.

Specifically, the study expects that layered deception will:

- increase engagement depth across multiple hosts

- influence attacker navigation through deception artifacts

- introduce delays in attacker progression toward higher-value assets

These effects will be evaluated using the behavioural metrics defined in this proposal.

# 4. System Architecture

The honeynet environment consists of three deception layers supported by monitoring and virtualization infrastructure.

Layer 1 – Network Deception

The first layer consists of network-accessible honeypot services deployed using an open-source honeypot platform.

These services simulate commonly targeted protocols such as SSH, web services, and file sharing.

This layer captures initial scanning activity and authentication attempts.

Layer 2 – Host-Level Deception

The second layer simulates an internal enterprise environment containing several host systems.

Example hosts include:

- workstation systems

- file servers

- administrative servers

- a domain controller

These hosts contain decoy credentials, documents, and configuration artifacts designed to encourage exploration and lateral movement attempts.

Layer 3 – Data-Level Deception

The third layer includes decoy data assets such as simulated cloud credentials and references to data storage resources.

These artifacts represent higher-value targets that attackers may attempt to access after interacting with earlier deception layers.

Monitoring and Logging Layer

A centralized monitoring node collects telemetry from honeypot services and host systems.

The monitoring system aggregates logs and converts them into structured records used for behavioural analysis.

Virtualization Infrastructure

The entire environment will be deployed on a Proxmox virtualization platform using approximately four to six virtual machines representing honeypots, enterprise hosts, and monitoring services.

Virtualization snapshots allow the environment to be reset between experiments.

## 4.1 Telemetry Instrumentation

To improve behavioural analysis, the system includes lightweight telemetry instrumentation that records interactions with deception artifacts and reconstructs attacker activity sequences.

Deception artifacts embedded in the environment will include unique identifiers that allow their usage to be tracked during analysis. Examples include decoy credentials, internal documentation, and cloud API keys.

When attackers interact with these artifacts, the telemetry system records the associated identifier together with the event details.

This allows the study to trace how specific artifacts influence attacker decisions.

## 4.2 Interaction Sequence Reconstruction

Collected telemetry will be normalized into structured event records representing transitions between hosts and services.

Example interaction sequence:

Attacker → Honeypot service  
Honeypot → Internal workstation  
Workstation → File server  
Workstation → Domain controller

Representing interactions as structured sequences allows the system to reconstruct attacker navigation paths through the environment.

## 4.3 Attacker Path Visualization

Interaction sequences may also be rendered as simple attacker path diagrams to improve interpretability of observed behaviour.

These visualizations illustrate how attackers move between deception layers and interact with decoy assets during a session.

Path visualizations will be used primarily as an analysis and reporting aid rather than as a separate analytical subsystem.

## 4.4 Deception Control Plane

The architecture introduces a Deception Control Plane that conditionally activates deeper deception layers based on observed attacker behaviour.

Instead of exposing all decoys simultaneously, additional hosts and artifacts may become visible only after specific trigger events occur.

Example trigger conditions include:

- successful honeypot login

- command execution on a compromised host

- interaction with a decoy credential

This staged activation simulates progressive discovery of infrastructure during an attack and allows researchers to observe how attackers respond to newly revealed resources.

# 5. Experimental Design

The honeynet environment will support several experimental scenarios designed to evaluate the behavioural effects of layered deception.

Scenario A – Network deception only

Only network honeypots are exposed.

Purpose: establish baseline attacker behaviour.

Scenario B – Network and host deception

Internal hosts and decoy credentials are introduced.

Purpose: observe exploration and lateral movement attempts.

Scenario C – Full layered deception

Data-level decoys such as cloud credentials are added.

Purpose: observe progression toward higher-value assets.

The experimental scenarios described in this section are designed to evaluate the behavioural hypotheses outlined earlier in the proposal.

# 6. Behavioural Metrics

Three behavioural metrics are used to evaluate the influence of layered deception on attacker activity.

Misdirection

Measures the number of interactions with deception artifacts such as decoy credentials or documents.

Delay

Measures the time attackers spend navigating the environment before reaching higher-value assets.

Engagement Depth

Represents the level of progression within the environment.

Example depth levels include:

| Level | Behaviour |
| --- | --- |
| 1 | scanning activity |
| 2 | authentication attempts |
| 3 | command execution |
| 4 | host exploration |
| 5 | credential discovery |
| 6 | lateral movement attempts |
| 7 | interaction with data-level decoys |

## 6.1 Evaluation Method

The evaluation compares attacker behaviour across the experimental scenarios described above.

Telemetry collected from the honeynet environment will be used to reconstruct interaction sequences and derive the behavioural metrics defined in this study.

The analysis examines whether layered deception results in observable differences in misdirection, delay, and engagement depth compared to the baseline scenario.

Selected sessions may also be visualized as attacker path diagrams to illustrate typical interaction patterns.

The results will be interpreted descriptively, focusing on behavioural patterns rather than statistical significance testing.

## 6.2 Threat Model

The honeynet environment is designed to observe the behaviour of opportunistic attackers and security practitioners interacting with exposed services.

The threat model assumes that attackers may perform activities such as:

- network scanning

- authentication attempts

- host exploration

- credential reuse

- access to data resources

The environment does not attempt to simulate highly targeted advanced persistent threat campaigns. Instead, it focuses on capturing behavioural patterns associated with attackers who interact with accessible services and explore the environment after gaining initial access.

# 7. MITRE ATT&CK Mapping

Observed activities will be mapped to high-level MITRE ATT&CK tactics.

Examples include:

| Behaviour | ATT&CK Tactic |
| --- | --- |
| network scanning | Reconnaissance |
| authentication attempts | Credential Access |
| command execution | Execution |
| host discovery | Discovery |
| credential reuse | Lateral Movement |
| data access attempts | Collection |

This mapping provides a structured framework for interpreting the captured behaviour.

# 8. Feasibility and Risk Management

The project remains feasible due to several design choices:

- limited environment size (four to six virtual machines)

- use of established open-source honeypot platforms

- virtualization snapshots enabling rapid environment reset

- emphasis on behavioural observation rather than complex detection systems

These choices ensure the project can be completed within the capstone timeframe.

# 9. Project Schedule and Learning Hours

| Milestone | Learning Hours |
| --- | --- |
| Literature review | 20 |
| Architecture design | 15 |
| Infrastructure setup | 15 |
| Network honeypot deployment | 20 |
| Host-level deception setup | 25 |
| Data-level deception setup | 15 |
| Telemetry integration | 15 |
| Experimental execution | 20 |
| Behaviour analysis | 20 |
| Final report preparation | 15 |

Total estimated learning hours: 180 hours

# 10. Expected Deliverables

The project will produce:

- a layered deception honeynet architecture

- a working virtualized deployment

- documented experimental methodology

- behavioural analysis of observed attacker interactions

- attacker path visualizations illustrating interaction patterns

- MITRE ATT&CK mapping of observed activities

# 11. Expected Contributions

This project contributes:

- a reproducible Deception-in-Depth honeynet architecture

- a structured methodology for observing attacker behaviour across multiple deception layers

- behavioural insights into how attackers interact with deceptive environments

The study emphasizes controlled experimentation and behavioural observation rather than large-scale operational deployment.

# Appendix A – Implementation Plan

## A.1 Purpose

This appendix provides a practical implementation plan for deploying the proposed Deception-in-Depth honeynet stack on the available Proxmox-based mini server with 32 GB RAM. It translates the proposal into a concrete execution layout, including virtual machines, resource allocation, network segmentation, and deployment sequence.

The design intentionally prioritizes manageability, reproducibility, and experimental usefulness over production-scale realism.

## A.2 Deployment Platform

The environment will be hosted on:

- Platform: Proxmox VE

- Available memory: 32 GB RAM

- Storage: local SSD / available local storage on the mini server

- Deployment model: isolated virtual lab

- Reset mechanism: Proxmox snapshots and rollback

Proxmox is suitable because it supports:

- multiple virtual machines

- isolated virtual networks

- easy snapshotting and rollback

- efficient local experimentation

This makes the environment appropriate for repeated behavioural experiments.

## A.3 Planned Virtual Machine Layout

The proposed environment will use 5 core virtual machines, with one optional sixth VM if resources remain stable during testing.

| VM Name | Role | Purpose | Suggested RAM | Suggested vCPU |
| --- | --- | --- | --- | --- |
| VM1-LOG01 | Monitoring / Logging Node | central log collection, basic dashboards, timeline reconstruction | 4 GB | 2 |
| VM2-TPOT01 | Network Honeypot Node | internet-facing network deception services | 8 GB | 4 |
| VM3-DC01 | Domain Controller | Active Directory services and identity-layer deception | 6 GB | 2 |
| VM4-WS01 | Employee Workstation | internal attacker exploration target with planted artifacts | 4 GB | 2 |
| VM5-FS01 | File / Internal Server | shared folders, decoy documents, credential artifacts | 4 GB | 2 |
| VM6-ADM01 (optional) | Admin / Jump / Decoy Server | additional lateral movement target and admin-themed artifacts | 2–4 GB | 2 |

Estimated baseline RAM usage

Without optional VM6:

- LOG01: 4 GB

- TPOT01: 8 GB

- DC01: 6 GB

- WS01: 4 GB

- FS01: 4 GB

Total: 26 GB

With optional VM6 at 2–4 GB:

Total: 28–30 GB

This leaves a small operating margin for Proxmox overhead and short-term resource spikes. For stability, the recommended default deployment is the 5-VM baseline, with the sixth VM enabled only after validation.

## A.4 Rationale for Feasibility

Although the project includes multiple hosts, the implementation remains manageable for several reasons.

Use of a small enterprise model

The environment is intentionally limited to a small enterprise simulation, not a full corporate lab. This is sufficient for observing:

- initial access

- internal discovery

- credential interaction

- lateral movement attempts

- data-decoy interaction

Reuse of established tools

The plan relies on existing open-source components rather than custom platform development. The main effort is integration and experimental design.

Controlled scope

Each VM serves a clear experimental role. There is no requirement to emulate every enterprise service.

Expandable but not dependent on expansion

The optional admin server improves realism, but the project does not depend on it. The baseline 5-VM layout is already enough for the proposed research questions.

## A.5 Proposed Network Layout

The environment should be segmented into three main logical networks.

| Network | Purpose | Connected VMs |
| --- | --- | --- |
| NET-EXT | attacker-facing / external services | TPOT01 |
| NET-INT | internal enterprise network | DC01, WS01, FS01, optional ADM01 |
| NET-MGMT | management and logging network | LOG01, Proxmox management access |

Network design principles

- NET-EXT exposes only the network deception layer.

- NET-INT simulates the internal enterprise environment.

- NET-MGMT is not exposed to the attacker and is used for administration, logging, and reset operations.

This design supports safe containment and clearer behavioural analysis.

## A.6 Logical Traffic Flow

The environment is designed to support a staged attacker journey.

Stage 1 – External interaction

Attackers first interact with services exposed on TPOT01.

Examples:

- SSH honeypot

- web honeypot

- selected service decoys

Stage 2 – Internal progression

If the attacker discovers or is presented with internal artifacts, the next likely targets are:

- WS01 for host exploration

- FS01 for file shares and decoy documents

- DC01 for identity-related discovery

Stage 3 – Higher-value decoys

If internal artifacts are followed successfully, attackers may reach:

- fake service credentials

- decoy internal references

- fake API keys or storage references

This staged layout directly supports the proposal’s measurement of:

- misdirection

- delay

- engagement depth

## A.7 Planned Role of Each VM

VM1 – LOG01

This system acts as the central monitoring and analysis node.

Planned functions:

- collect logs from honeypots and hosts

- store authentication and access records

- support manual timeline reconstruction

- support ATT&CK-oriented analysis

This VM is feasible because the project only requires basic centralized telemetry, not a heavy enterprise SIEM deployment.

VM2 – TPOT01

This is the entry-facing deception node.

Planned functions:

- host T-Pot CE services

- capture scans and login attempts

- log initial attacker interaction

This is one of the more resource-intensive VMs, which is why it is assigned the highest RAM allocation.

Manageability is still reasonable because it consolidates multiple network deception services into one host.

VM3 – DC01

This system provides a minimal Active Directory environment.

Planned functions:

- domain services

- user and service account simulation

- selected deceptive AD objects or relationships

This component may appear challenging, but it is manageable because the plan uses GOAD as the base, reducing setup complexity significantly.

VM4 – WS01

This workstation supports host-level observation.

Planned functions:

- simulate a user endpoint

- contain planted files and decoy notes

- provide a believable exploration target

This VM does not require heavy services, so moderate resources are sufficient.

VM5 – FS01

This system acts as the internal resource server.

Planned functions:

- host shared folders

- contain decoy documents

- expose planted credentials or references

- encourage attacker browsing and movement

This VM is strategically important because it can naturally bridge host deception and data-level deception.

VM6 – ADM01 (optional)

This optional VM improves realism and supports additional lateral movement paths.

Possible functions:

- admin-themed scripts

- backup-related decoys

- extra credential artifacts

- additional decision points for attacker path selection

This VM should be deployed only if memory usage remains stable after baseline testing.

## A.8 Deception Asset Placement Plan

To make the system experimentally useful, decoy assets should be distributed across the environment.

On TPOT01

- exposed service banners

- realistic login prompts

- basic attacker-facing lure points

On WS01

- admin notes

- mapped drive references

- configuration files

- workstation artifacts suggesting internal systems

On FS01

- shared folders

- documents with fake credentials

- references to higher-value resources

- cloud-style credential artifacts

On DC01

- believable user accounts

- selected service accounts

- identity objects supporting enterprise realism

Data-level decoys

- fake API keys

- decoy bucket names

- monitored references to cloud resources

To improve analysis quality, each major decoy should have a unique identifier so that later it is possible to trace which artifact influenced attacker behaviour.

## A.9 Implementation Sequence

The deployment should follow a staged sequence to reduce risk and troubleshooting complexity.

Phase 1 – Base infrastructure

- create Proxmox virtual networks

- deploy LOG01

- deploy TPOT01

- confirm isolated external-facing deception works

Phase 2 – Internal enterprise setup

- deploy DC01

- deploy WS01

- deploy FS01

- verify internal connectivity and domain operation

Phase 3 – Deception artifact placement

- add fake credentials

- add decoy files and internal references

- add selected admin-themed artifacts

Phase 4 – Data-level decoys

- add fake API keys

- add cloud-style references

- configure monitoring for decoy credential usage

Phase 5 – Logging and validation

- forward logs to LOG01

- verify event correlation across VMs

- test reconstruction of simple attacker paths

Phase 6 – Experiment preparation

- snapshot the environment

- document baseline state

- run Scenario A / B / C experiments

This phased approach makes the project manageable because each stage can be validated before moving to the next.

## A.10 Snapshot and Reset Strategy

A key part of feasibility is the ability to restore the lab quickly.

Planned reset method

- take a clean snapshot after stable deployment

- take an experiment-ready snapshot after deception assets are placed

- restore to the experiment-ready state between runs

This reduces the operational burden of rebuilding systems manually and makes multiple experimental scenarios realistic within capstone time constraints.

## A.11 Key Manageability Considerations

Resource constraint

The most likely limitation is RAM pressure from running multiple VMs simultaneously.

Mitigation: 
Use the 5-VM baseline first, keep ADM01 optional, and avoid overloading LOG01 with heavyweight tooling.

AD lab complexity

Domain-based environments can be difficult to build manually.

Mitigation: 
Use GOAD as the base and limit modifications to deception-oriented changes only.

Logging complexity

Centralized telemetry can become hard to manage if the scope is too broad.

Mitigation: 
Collect only the logs needed for the proposal metrics and ATT&CK-aligned interpretation.

Experiment reproducibility

Without reset capability, repeated tests may become inconsistent.

Mitigation: 
Use Proxmox snapshots as part of the core workflow.

## A.12 Summary

The proposed implementation plan is feasible on the available 32 GB Proxmox mini server because it uses:

- a limited number of VMs

- a small-enterprise simulation model

- established open-source frameworks

- controlled deception placement

- snapshot-based reset

The design is sufficiently rich to support the project’s research goals, while still remaining realistic for a capstone-level implementation.

# Appendix B – Experimental Telemetry Schema

## B.1 Purpose

To support behavioural analysis of attacker activity within the honeynet environment, the project introduces a structured telemetry schema for recording and analysing observed interactions.

While the honeynet components generate raw logs from multiple sources (such as honeypot services, host systems, and monitoring tools), these logs can be difficult to interpret directly. The telemetry schema defined in this appendix provides a normalized format that enables consistent analysis of attacker behaviour across deception layers.

The schema supports several analysis objectives described in the proposal, including:

- reconstruction of attacker interaction sequences

- identification of interactions with deception artifacts

- measurement of behavioural metrics such as misdirection and engagement depth

- generation of attacker path visualizations

The schema is designed to remain lightweight and feasible within the scope of the capstone project while providing sufficient structure for meaningful behavioural analysis.

## B.2 Logging Architecture Overview

Telemetry collected from the honeynet environment will be processed through a simple normalization pipeline.

Raw logs generated by deception services and host systems will first be collected by the centralized monitoring node. A lightweight parsing script will then extract relevant information and convert it into structured records following the telemetry schema.

The resulting datasets will be stored locally in formats suitable for analysis, such as CSV files or a lightweight SQLite database.

The telemetry pipeline can be summarized as follows:

Raw Honeynet Logs (T-Pot, host logs, control-plane events) │ ▼ Log Normalization Script │ ▼ Structured Telemetry Records (session, events, artifacts, interaction edges) │ ▼ Analysis and Visualization (metrics, interaction graphs, attacker paths)

This approach allows the project to preserve original logs while enabling structured behavioural analysis.

## B.3 Core Telemetry Datasets

The telemetry schema consists of several structured datasets that capture different aspects of attacker interaction.

The primary datasets include:

- Session Log – tracks individual attacker sessions

- Event Log – records normalized security events

- Artifact Interaction Log – records interactions with deception artifacts

- Interaction Edge Log – records transitions between hosts and services

- Control Plane Trigger Log – records activation of deception layers

Together, these datasets provide a comprehensive view of attacker activity within the honeynet environment.

## B.4 Session Log

The session log tracks each observed attacker interaction session.

A session represents a sequence of related actions originating from the same source actor or IP address. Grouping events into sessions allows the system to measure session duration and reconstruct attacker activity timelines.

Example fields include:

| Field | Description |
| --- | --- |
| session_id | unique identifier for the attacker session |
| actor_id | identifier for the observed attacker |
| source_ip | originating IP address |
| entry_point | first service contacted (e.g., SSH honeypot) |
| scenario_id | experimental scenario identifier |
| first_seen_ts | timestamp of first observed event |
| last_seen_ts | timestamp of last observed event |
| session_status | session state (active, closed, timeout) |

Session data allows the project to evaluate metrics such as interaction duration and engagement depth.

## B.5 Event Log

The event log records normalized security events derived from honeypot logs and host telemetry.

Each record represents a single observed action, such as a login attempt, command execution, or file access event.

Typical fields include:

| Field | Description |
| --- | --- |
| event_id | unique identifier for the event |
| session_id | linked attacker session |
| timestamp | time of the event |
| event_type | normalized event category |
| source_node | host or actor initiating the event |
| target_node | system or service targeted |
| username | account name used, if applicable |
| artifact_id | identifier of related deception artifact |
| protocol | communication protocol |
| action | concise description of activity |
| result | outcome of the action |
| attack_tactic | mapped MITRE ATT&CK tactic |

Examples of normalized event types include:

- network scan

- login attempt

- login success

- command execution

- file access

- credential usage

- API key access

The event log forms the foundation for behavioural analysis and MITRE ATT&CK mapping.

## B.6 Artifact Interaction Log

To improve interpretability of attacker decisions, deception artifacts will include unique identifiers that allow their usage to be tracked.

Examples of artifacts include:

- decoy credentials

- configuration files

- internal documentation

- cloud API keys

- references to internal systems

Example artifact identifiers:

| Artifact Type | Example Identifier |
| --- | --- |
| credential | svc_backup_pw_A17 |
| document | finance_archive_note_B04 |
| API key | aws_key_C09 |
| host reference | fileserver_ref_D02 |

When attackers interact with these artifacts, the system records an artifact interaction event.

Typical fields include:

| Field | Description |
| --- | --- |
| artifact_event_id | unique artifact event identifier |
| session_id | associated attacker session |
| timestamp | event time |
| artifact_id | identifier of the deception artifact |
| artifact_type | artifact category |
| host | host where artifact interaction occurred |
| interaction_type | type of interaction (opened, copied, used) |
| resulting_target | next host or resource accessed |

Artifact interaction logging allows the project to trace causal relationships between deception artifacts and attacker navigation decisions.

## B.7 Interaction Edge Log

The interaction edge log represents transitions between hosts or services as structured relationships.

Instead of viewing attacker behaviour as isolated events, this dataset represents the activity as a sequence of directed interactions.

Example fields include:

| Field | Description |
| --- | --- |
| edge_id | unique edge identifier |
| session_id | associated attacker session |
| timestamp | time of interaction |
| source_node | source system or actor |
| target_node | destination system or service |
| interaction_type | type of interaction |
| artifact_id | linked artifact identifier if applicable |
| outcome | result of interaction |
| depth_level | calculated engagement depth |

These edges can be used to reconstruct attacker navigation paths and generate interaction graphs.

## B.8 Control Plane Trigger Log

Because the architecture includes a Deception Control Plane that conditionally activates deeper deception layers, trigger events will also be recorded.

Example fields include:

| Field | Description |
| --- | --- |
| trigger_id | unique trigger identifier |
| timestamp | time of activation |
| session_id | related attacker session |
| trigger_condition | event that caused activation |
| action_taken | action performed by control plane |
| affected_asset | host or resource activated |
| result | trigger execution status |

These records allow researchers to correlate attacker behaviour with dynamic changes in the deception environment.

## B.9 Attacker Path Reconstruction

Using the interaction edge dataset, attacker activity can be reconstructed as sequential paths through the honeynet environment.

Example interaction path:

Attacker │ ▼ TPOT01 │ ▼ WS01 ↙ ↘ FS01 DC01

These reconstructed paths help illustrate how attackers navigate between deception layers and interact with decoy assets.

In the final report, selected sessions may be visualized as attacker path diagrams to demonstrate typical behavioural patterns observed during experiments.

## B.10 Alignment with Behavioural Metrics

The telemetry schema supports the behavioural metrics defined in the main proposal:

| Metric | Supporting Data |
| --- | --- |
| Misdirection | artifact interaction log |
| Delay | session log and event timestamps |
| Engagement Depth | interaction edge log |
| Interaction sequences | event and edge datasets |

This structured logging approach ensures that the experimental results can be analysed systematically while remaining feasible within the scope of the capstone project.

# Appendix C – Telemetry Processing Pipeline

## C.1 Purpose

To support behavioural analysis within the deception-in-depth honeynet environment, the project includes a lightweight telemetry processing pipeline that converts raw logs generated by honeypot services and host systems into structured datasets.

Honeypot frameworks and operating systems typically produce large volumes of raw log entries that are difficult to interpret directly. The telemetry pipeline addresses this limitation by extracting relevant security events and converting them into normalized records following the telemetry schema described in Appendix B.

This pipeline enables the project to reconstruct attacker interaction sequences, identify interactions with deception artifacts, and generate behavioural metrics used in the evaluation of the honeynet architecture.

The pipeline is intentionally designed to remain lightweight and manageable within the scope of the capstone project, while still providing sufficient structure for systematic behavioural analysis.

## C.2 Pipeline Architecture

The telemetry processing pipeline follows a simple sequential architecture consisting of four main stages:

- Log collection

- Event normalization

- Session and artifact enrichment

- Structured data export

The processing flow is illustrated below.

Raw Honeynet Logs (T-Pot, host systems, control plane events) │ ▼ Log Parsers (extract relevant security events) │ ▼ Event Normalization (convert logs into structured event records) │ ▼ Session & Artifact Enrichment (session assignment, artifact identification) │ ▼ Interaction Edge Generation (reconstruct attacker navigation paths) │ ▼ Structured Telemetry Outputs (session_log, event_log, artifact_log, edge_log)

This architecture allows raw logs to remain available for reference while providing normalized datasets suitable for behavioural analysis.

## C.3 Log Sources

The telemetry pipeline processes logs generated from multiple components within the honeynet environment.

Typical log sources include:

| Source | Example Data |
| --- | --- |
| Network honeypots | login attempts, service probes |
| Host systems | authentication events, command execution |
| File shares | file access activity |
| Deception artifacts | credential or document usage |
| Deception control plane | activation triggers |

These logs are collected by the centralized monitoring node before being processed by the normalization pipeline.

## C.4 Event Normalization

During normalization, raw log entries are converted into standardized event records that share a consistent structure.

Each event record captures key information about the observed activity, including:

- event timestamp

- event type

- source actor or system

- target host or resource

- username or credential used

- related artifact identifier (if applicable)

- outcome of the action

Normalization ensures that events originating from different log sources can be analysed using the same schema.

This step simplifies subsequent analysis and allows behavioural patterns to be compared across different experiment scenarios.

## C.5 Session Identification

After normalization, events are grouped into attacker sessions.

A session represents a sequence of related actions originating from the same source actor or IP address within a defined time window.

Session grouping allows the project to:

- reconstruct attacker activity timelines

- measure session duration

- analyse engagement depth across the environment

A simple inactivity threshold is used to separate sessions when a source actor becomes inactive for a defined period of time.

## C.6 Artifact Identification

As described in Appendix B, deception artifacts embedded in the environment include unique identifiers.

Examples of artifacts include:

- decoy credentials

- internal documents

- configuration notes

- API keys

- references to internal hosts

When these identifiers appear within event records, the telemetry pipeline associates the corresponding artifact identifier with the event.

This allows the system to track how interactions with specific artifacts influence attacker behaviour and navigation decisions.

## C.7 Interaction Edge Generation

To support reconstruction of attacker navigation paths, the telemetry pipeline converts normalized events into interaction edges.

An interaction edge represents a transition between two entities within the environment, such as:

- attacker → honeypot service

- compromised host → internal server

- workstation → domain controller

Each edge includes information describing the interaction type and the depth of engagement within the environment.

These edges allow the project to model attacker activity as a sequence of directed interactions rather than isolated log events.

## C.8 Structured Data Export

After processing, the telemetry pipeline exports structured records into datasets corresponding to the schema described in Appendix B.

These datasets include:

| Dataset | Purpose |
| --- | --- |
| session_log | attacker session tracking |
| event_log | normalized event records |
| artifact_log | interactions with deception artifacts |
| interaction_edge_log | attacker navigation paths |
| control_plane_log | activation of deception layers |

The datasets may be stored in simple formats such as CSV files or a lightweight SQLite database to facilitate analysis and visualization.

## C.9 Analysis and Visualization

The structured telemetry outputs allow the project to perform several forms of behavioural analysis, including:

- reconstruction of attacker activity timelines

- measurement of engagement depth

- identification of interactions with deception artifacts

- generation of attacker path visualizations

For example, an observed session may produce a path similar to the following:

Attacker │ ▼ TPOT01 │ ▼ WS01 ↙ ↘ FS01 DC01

Such visualizations help illustrate how attackers navigate between deception layers and interact with decoy assets within the environment.

These reconstructed paths will be used in the final report to demonstrate behavioural patterns observed during the experiments.

## C.10 Implementation Considerations

The telemetry pipeline will be implemented as a lightweight script executed on the monitoring node.

The implementation will focus on:

- simple log parsing

- minimal dependencies

- straightforward data export

Because the pipeline relies primarily on log processing and basic data transformation, the required implementation effort remains modest and suitable for the scope of a capstone project.

This design ensures that the telemetry processing component enhances the research value of the honeynet environment while remaining practical to implement within the project timeframe.